# ESP32 Pin Connections

## 1. MPU6050 (I2C)

| MPU6050 Pin | Connect To (ESP32) |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| SDA | GPIO 21 |
| SCL | GPIO 22 |

⚠️ Important: MPU6050 operates at 3.3V logic. Do not connect it to 5V on VCC.

## 2. Ultrasonic Sensor (HC-SR04)

| HC-SR04 Pin | Connect To (ESP32) |
| --- | --- |
| VCC | 5V |
| GND | GND |
| Trig | GPIO 2 |
| Echo | GPIO 4 |

## 3. Motor Driver (L298N)

| L298N Pin | Connect To (ESP32) | Notes |
| --- | --- | --- |
| IN1 | GPIO 32 | Left Motor Direction 1 |
| IN2 | GPIO 33 | Left Motor Direction 2 |
| IN3 | GPIO 18 | Right Motor Direction 1 |
| IN4 | GPIO 19 | Right Motor Direction 2 |
| ENA | PWM Channel 5 | Use ledcAttach(PWM_CH_ENA, ...) |
| ENB | PWM Channel 15 | Use ledcAttach(PWM_CH_ENB, ...) |
| GND | GND (shared) | Shared with ESP and Battery GND |
| VCC (12V) | Battery + (6–12V) | For motors only |
| VCC (5V) | Vin | For ESP |